Lab 4 Report — Robot Localisation with Particle Filter and Evolutionary Parameter Search

Student: Ferdinand Cordonnier
Environment: PyBullet + Husky robot + maze map (maze_realistic.txt)
Goal: Localise the robot in the maze using lidar-like scans + particle filter, and automatically search for strong parameter sets under time constraints.

1. Context and Objective

Lab 4 focuses on the localisation problem: estimating the robot’s pose xyθwhile it navigates in a maze-like environment. The robot receives motion commands and obtains periodic lidar-like distance measurements. The primary algorithm used is a Particle Filter (PF).

A key difficulty is that PF performance depends strongly on many parameters (number of particles, scan frequency, lidar ray count, physics settings, motion speed, etc.). Because the parameter space is high-dimensional and non-linear, I implemented an automated evolutionary optimisation tool to systematically search for parameter configurations that yield the best accuracy under a strict time budget.

2. High-Level System Architecture

The system is designed as three layers:

- Simulation Layer (PyBullet)

- Localisation Layer (Particle Filter)

- Evaluation & Optimisation Layer (Evolutionary Search + CSV-based scoring)

Structural Architecture Diagram (copy-paste)

┌───────────────────────────────────────────────────────────────────┐

│               Evaluation & Optimisation Layer                      │

│                                                                   │

│  Evolutionary (Genetic-inspired) Optimiser                         │

│   - Population of parameter sets                                   │

│   - Fitness evaluation (RMSE, final error, convergence, time)       │

│   - Selection (elitism)                                             │

│   - Mutation (local search around best)                             │

│   - Limited random exploration (avoid local minima)                 │

│                                                                   │

│  Reads run outputs (pf.csv) → computes fitness                      │

└───────────────────────────────▲───────────────────────────────────┘

│

│ CSV logs (pf.csv, odometry.csv)

│

┌───────────────────────────────┼───────────────────────────────────┐

│               Experiment Orchestration Layer                        │

│                                                                   │

│  - Fixed motion scenario (autotest)                                 │

│  - Time-bounded run (≤ 30–45 s)                                     │

│  - Launch mode: GUI for debug / DIRECT for batch optimisation       │

└───────────────────────────────┼───────────────────────────────────┘

│ commands (v,w), scan schedule

│

┌───────────────────────────────┼───────────────────────────────────┐

│                 Localisation Layer (Particle Filter)                │

│                                                                   │

│  PF loop: predict → update → resample                               │

│   - Predict: propagate particles using commanded motion             │

│   - Update: compare real scan vs simulated scan per particle        │

│   - Resample: triggered via Neff threshold                          │

│  Output: estimated pose + error metrics                             │

└───────────────────────────────┼───────────────────────────────────┘

│ physics + sensing

│

┌───────────────────────────────┼───────────────────────────────────┐

│                 Simulation Layer (PyBullet)                         │

│                                                                   │

│  - Maze from grid map (maze_realistic.txt)                          │

│  - Husky robot model                                                │

│  - Lidar-like raycasting (K rays)                                   │

│  - Configurable physics (friction, timestep frequency)              │

└───────────────────────────────────────────────────────────────────┘

3. Simulation Layer (PyBullet)

3.1 World and Map

- The maze is loaded from a text grid map file (maze_realistic.txt).

- Each wall cell is converted into physical obstacles in PyBullet.

- A consistent cell size is used to map grid coordinates to world coordinates.

3.2 Robot

- The robot is the Husky model, controlled via commanded linear/angular velocities vω.

- The robot is spawned automatically in a free area near the top of the map to ensure collision-free initialisation.

3.3 Physics Parameters

- The simulation uses a stable timestep (--hz) and can run in:

  - GUI mode for debugging and visual validation

  - DIRECT mode for fast automated runs

- A global friction coefficient can be applied (--friction) to investigate how wheel-ground interaction influences motion realism and localisation stability.

4. Sensor Model: Lidar-like Scans

Instead of using a heavy sensor model, I implemented a lightweight lidar approximation using ray casting:

- A scan consists of K rays (--angles_n), evenly distributed around the robot.

- Each ray returns the distance to the closest obstacle in that direction.

- Scans are performed at a configurable interval (--scan_period).

This design allows tuning the trade-off between:

- Information quality (more rays, more frequent scans)

- Computation cost (fewer rays, slower scan rate)

5. Localisation Layer: Particle Filter

5.1 State Representation

Each particle represents a pose hypothesis:

pi=(xi,yi,θi)

with an associated weight wi.

5.2 Prediction Step

Particles are propagated using commanded velocities:

- dC=v⋅dt

- dT=ω⋅dt

This acts as a simplified odometry model. It is fast and stable, and it ensures a consistent prediction model across all tests.

5.3 Measurement Update

At each scan time:

- A real scan zis obtained from the robot pose.

- For each particle pi, a simulated scan ziis computed.

- Weights are updated based on scan similarity:

- particles producing scans closer to zreceive higher weights.

5.4 Resampling

Resampling is triggered when the effective number of particles Neffdrops below a threshold (e.g., 35% of N), preventing degeneracy.

5.5 Initialisation Strategy

Particles are initialised using a Gaussian prior around the spawn position and then filtered to ensure particles are not placed inside walls. Invalid particles are resampled near the spawn (not randomly across the whole map), improving early stability and avoiding global divergence.

6. Experiment Orchestration and Reproducibility

6.1 Fixed Motion Scenario (Autotest)

To ensure all configurations are evaluated fairly, I created a deterministic test scenario:

- Drive straight until ~5 meters are travelled

- Rotate left by 90° on the spot

- Drive while turning left for a limited duration (≈ 10–15 seconds, can be shortened)

Each run is time-bounded (≤ 30–45 s), ensuring controlled cost and enabling many repeated evaluations.

6.2 Logging

The program logs data to CSV files per run, including:

- estimated pose (PF)

- ground truth pose

- localisation error (err_xy)

- effective sample size (neff)
This makes analysis independent from the simulator itself and supports batch optimisation.

7. Automated Parameter Search Software (Evolutionary / Genetic-Inspired)

7.1 Motivation for Evolutionary Search

Manual tuning is inefficient because:

- the parameter space is large

- parameters interact non-linearly (example: scan rate interacts with ray count and particle count)

- the system is stochastic (PF + physics)

- the objective is multi-criteria: accuracy AND speed

A genetic-inspired evolutionary approach is well suited because it:

- does not require gradients

- handles noisy evaluations

- can optimise under time budgets

- balances exploration and exploitation naturally

7.2 Parameters Considered

The optimiser searches combinations of:

- PF parameters: n (particle count)

- sensor parameters: angles_n (ray count), scan_period

- simulation parameters: timestep frequency (hz)

- physics: friction

- motion scenario parameters: speeds (straight/turn/curve) and curve duration

7.3 Fitness Function

Each candidate is scored using lightweight analysis of the generated pf.csv, using:

- RMSE position error

- final position error

- convergence indicators (optional)

- execution time penalty (so faster configs are preferred if accuracy is similar)

This allows systematic ranking of configurations.

7.4 Selection and Mutation Strategy

- Selection (elitism): keep the top-performing individuals

- Mutation: small perturbations around the best candidates to refine performance

- Limited random exploration: prevents getting stuck in local minima

A “narrowed search” mode was implemented after initial exploration to focus on promising zones (for example limiting rays to 16–24 and scan_period to ~0.6–1.0 seconds), leading to more consistent improvements.

8. Results (Current Status)

With automated optimisation under time constraints, the best configurations obtained achieve:

- RMSE ≈ 1.5 m (typical best achieved so far)

- stable tracking behaviour (error does not diverge immediately)

- improvement trends when increasing rays (up to 24) and choosing scan periods around ~0.8–1.0 s

While strict convergence below 0.5 m is not consistently achieved across all conditions, the experiments are reproducible and the optimisation framework provides objective evidence and justification for selected parameters.

9. Discussion and Trade-offs

The primary limitation is the trade-off between:

- improving accuracy (more particles, more rays, faster scans)

- keeping cost low (time-bounded execution)

The evolutionary tool is valuable because it:

- quantifies this trade-off with real data

- provides a systematic way to justify parameter choices

- avoids “trial-and-error” tuning and supports a rigorous methodology

10. Conclusion

This work delivers:

- A complete PF localisation pipeline for a Husky robot in a maze environment

- A reproducible autotest scenario and CSV-based evaluation pipeline

- A dedicated evolutionary optimisation tool to automatically identify good parameter sets under strict timing constraints

The optimisation tool is a major contribution because it transforms parameter tuning into a structured, measurable engineering process.